This test exposes a potential priority inversion when priority inheritance is

used and multiple locks are acquired by a low priority task. The scenario

consists of 2 tasks of different priority and 2 semaphore/mutexes using

priority inheritance. The scenario is:

Task 1 has priority 36.  Task 2 has priority 34.

1. Task 1 obtains semaphore S1.

2. Task 1 obtains semaphore S2

3. Task 2 preempts Task 1.

4. Task 2 blocks on S2.  Task 1 inherits priority 34.

5. Task 1 resumes. Task 1 releases S2. Task 1 continues to run at priority 34.

6. Task 1 releases S1. Task 1 changes priority back to 36.

7. Task 2 now preempts Task 1 and begins operating again.

During steps 5-6 Task 1 executes when Task 2 should be able to execute.

Stepping down the priority of Task 1 when it releases S2 back to its original

priority when it obtained S2 fixes the priority inversion.